# Cable Harness Tester - 8HV 50 Signal Line Technical Report

# 1. Executive Summary

This report outlines the design and implementation of an industrial-grade Cable Harness Tester. The project transitioned from a compact prototype to a high-density professional industrial architecture to ensure durability, scalability, and ease of manufacturing. All the client's original electrical and mechanical requirements have been fully integrated.

# 2. Project Requirements

Signal Capacity: Support for over 50 individual signal lines for cable testing.

Protection: 24V surge protection on every signal line (Industrial Standard).

MCU: ESP32-WROOM-32E (Integrated WiFi/Bluetooth).

HV Input: Support for 24V High-Voltage monitoring inputs.

Industrial Connectors: Use of pluggable 3.5mm Screw Terminals (Phoenix Contact Style).

Mechanical: DIN-Rail compatible mounting system.

Aesthetics: Professional layout with perimeter-aligned components.

# 3. Technical Hardware Implementation

## 3.1 Architecture & Components

The hardware utilizes the following key components:

Board Size: 160mm x 100mm (Industrial Eurocard Standard).

I/O Expansion: 4x MCP23017 I2C Expanders (providing 64 total I/O ports).

Series Protection: 50x 330Ω Resistors to limit current.

Overvoltage Protection: 50x TVS Diodes (3.3V breakdown) for surge suppression.

ADC Scaling: 90.9kΩ + 10kΩ voltage dividers for 24V -> 2.9V signal scaling.

Microcontroller: ESP32-WROOM-32E with cleared antenna keep-out zone.

Human Interface: On-board tactile reset/boot buttons and status indicators.

# 4. Professional Layout (Waterfall Architecture)

To move away from a 'hobbyist' look, a Waterfall Architecture was implemented:

Perimeter ALignment: All terminal blocks (J1-J8) are aligned at the extreme top (Y=3.5mm) and bottom (Y=96.5mm) of the PCB. This provides maximum space for external wire plugs.

Component Pipeline: Components are arranged in logical vertical columns (Waterfall columns). Signals flow linearly from the MCP Chips -> Resistors -> Diodes -> Connectors.

RF Implementation: The ESP32 is placed on the far right edge of the board with the antenna facing OUTWARD. This ensures zero copper interference from the 24V layers.

Mechanical Rigidity: Four M3 mounting holes are located at the corners (Inset 4mm) for secure industrial mounting.

# 5. Requirement Completion Status

Board Dimensions (160x100mm): COMPLETE

50-line TVS/Resistor Protection: COMPLETE

ESP32 RF Edge Placement: COMPLETE

Hobby Goldpin Removal: COMPLETE

Industrial Terminal Block Positioning: COMPLETE

BOM & Pick and Place Export: COMPLETE

# 6. Conclusion

The project has been successfully completed according to all specified professional standards. The design files are ready for routing and production. The use of specialized industrial placements ensures a reliable product with long-term field stability.

# 8. Visual Component Identification Guide

This section provides a visual reference for the components as seen in the 3D and Schematic views of the professional 160x100mm design.

## 8.1 3D PCB View Identification

Heading: Microcontroller Unit (U1) - Located on the far left edge to ensure WiFi antenna clearance.

Heading: I/O Expansion Cluster (U2-U5) - Centrally located 4x MCP23017 ICs handling the 50+ signal logic.

Heading: Protection Matrix - High-density grid of SMD 330Ω resistors and TVS diodes located on the right logic channel.

Heading: Industrial Interface - Pluggable screw terminal blocks positioned along the perimeter for external harness connection.

Heading: Power & Control Zone - Top right corner containing the 24V input terminal, status LEDs, and tactile control buttons.

## 8.2 Schematic Architecture Identification

Heading: Protection Channel - The repeated R-D (Resistor-Diode) pairs providing 24V surge safety.

Heading: I2C Control Bus - The shared SDA/SCL lines connecting all 4 expansion chips to the ESP32.

Heading: 24V ADC Scaling - The voltage divider network (90.9kΩ/10kΩ) preparing high-voltage signals for the MCU logic levels.